Street Improvement

General Notes

1. All work shall be done in accordance with this plan and the current standard specifications of the City of Chino Hills Engineering Department.

2. Standards and Specifications for transportation facilities and all work within the public right of way shall conform to the City’s Standards and Specification Manual unless specifically modified herein, augmented by the Standard Specifications for Public Works Construction, 2006 Edition.

3. Standard Specifications cover the usual construction requirements for work specified on a city permit; however, in the event it is determined that the specific work to be done is of such a nature that the method of construction, type and/or kind of material is not defined by the Standard Specifications, such work shall be performed in accordance with special provisions prepared for the specific work and made a part of the approved plans.

4. Where construction plans have been prepared by other than the City Engineer’s staff, all lines and grades will be set and established under the direction of the private engineer who prepared the plans or an engineer or surveyor acting on his behalf.

a. Adequate stakes shall be set by the private engineer to enable the contractor to construct the work to the plan grades.

b. Stakes shall not be destroyed until the work has been reviewed and approved by the public works inspector.

c. Subgrade stakes, driven to the pavement subgrade elevation, shall be set by the contractor or private engineer. The stakes shall be set at all hinge, edge of pavement and crown points within 0.01 foot of subgrade elevation and not more than 50 foot intervals. Additional stakes may be needed at warped or vertical curve areas. Subgrade stakes shall be in place prior to paving to permit spot checking of subgrade by the public works inspector.

5. All sewer, water, electric, gas, phone or other utility laterals, mains or crossings shall be constructed prior to paving.

6. The materials to be furnished and used in this work shall be controlled in accordance with the provisions of Section 6 of the Standard Specifications. Testing of materials shall be in accordance with the schedule in Section 6(D). Written reports of these tests and a plat showing locations, shall be submitted by the testing laboratory to the Public Works Inspector prior to proceeding with the next construction operation. If other manufactured products are used, a certificate of compliance from the manufacturer may be provided to verify the quality of materials unless otherwise required by the plans.

7. The hours of operation shall be from 6:00 AM to 7:00 PM, Monday through Friday. Other hours shall require written authorization from the City Engineer or City Building Official.

8. No work shall be undertaken without obtaining all necessary permits from the City. The site superintendent or foreman shall notify the Engineering Inspector  (909) 364-2770 at least 48 hours in advance of any inspections required.

9. State Law SB 3019 requires the contractor to obtain an identification number from Underground Service Alert prior to the issuance of a city encroachment permit. The contractor shall notify Underground Service Alert two full working days (48 hours minimum) in advance of any construction activities, including pavement removal, excavation and AC overlay, which could affect any underground utility.

10. Any contractor performing work on this project shall familiarize himself with the site and shall be responsible for any damage to existing facilities resulting directly or indirectly from his operations, whether or not such facilities are shown on the plans.

11. Prior to any construction work which could damage or conflict with underground structures or utilities, the contractor shall excavate inspection holes to determine the location and depth of said structures and utilities in the vicinity of, or which may be affected by, the proposed improvement work.

12. If existing utilities or other facilities conflict with the proposed improvements, work shall stop and the engineer be notified immediately.

13. The contractor shall repair or replace any public improvements damaged as a result of construction as required by the Public Works Inspector.

14. The site shall be kept regularly watered to mitigate blowing dust for the duration of the high winds.

15. No water shall be obtained from any City fire hydrant without the use of an authorized hydrant meter.

16. Survey monuments shall be protected and perpetuated in place. Any disturbed or covered monuments shall be reset by a Registered Civil Engineer or Licensed Land Surveyor.

17. New and existing manholes and valve cans shall be adjusted to grade.

18. Trench backfill shall be done in accordance with City Standard Drawing No. W-3.

19. The thickness of base material shall be determined by soils tests and recommendations from the soils engineer. Upon completion of rough grading, R-values, and sand equivalents, the structural section of streets shall be submitted to the City Engineer for approval.

20. The contractor shall remove and replace, in kind plus one (1”) inch, a minimum of two feet in width of existing asphalt concrete and aggregate base adjacent to newly constructed curb and gutter.

21. The final surface layer of asphalt concrete (0.12’ min.) shall not be placed until all onsite improvements, including grading have been completed and all unacceptable work has been corrected unless otherwise approved by the Public Works Inspector.

22. The contractor shall remove and install pavement markings and striping as required by the Public Works Inspector in the field. This requirement shall include the pavement markers described in the hydrant specifications.

23. “As-built” record drawings shall be furnished to the city upon completion of the project.

24. Approval of these plans by the City or its agents shall not relieve the contractor or the applicant of the responsibility for correction of errors or omissions discovered during construction. Upon request, the appropriate plan revisions shall be promptly submitted to the City Engineer for review and approval.

25. Estimate of quantities is provided by the engineer only for the convenience of the developer. The contractor shall make his own determination of construction quantities before submitting a bid. Any item of work required by these plans which is not specifically listed in the estimate of quantities shall be considered as included in the other items of work.

